CRITICAL: CHANGE FONT BACK TO 11PT for peer review!

Article Title (110 characters or less)

First Author1, Second Author2, Third Author3

1 Department (if applicable), Institution/School, City, State, Country (if non-US)

2 Department (if applicable), Institution/School, City, State, Country (if non-US)

3 Department (if applicable), Institution/School, City, State, Country (if non-US)

Student Authors

First Student Author, Middle/High School

Second Student Author, Middle/High School

KEYWORDS: Accessibility, Wearables, Stereoscopy, Audio, Transformers

OVERVIEW: Provide a 2- to 3-sentence overview of your manuscript. This will be used as a blurb for your article when posting the final publication on our website.

SUMMARY

Visually impaired individuals often face a lot of problems every day regarding navigation, understanding visual scenes and environments, object localization and reading printed or handwritten text. One of the most frustrating issues that they face is depth perception, as it is directly tied with independent navigation and avoiding hazards. These daily difficulties and limitations of vision put up a barrier to how they interact, how easily they can fetch household objects and how they perceive dynamic and moving bodies. To address these issues, we developed Pixelsense, an AI-powered wearable assistive technology designed to map the depth of surroundings through stereoscopic cameras. We hypothesized that vision can be supplemented by transforming surrounding scene into depth map and representing them through time-synchronized spatial audio. We conducted multiple distinct experiments to evaluate the effectiveness of the system. We assessed the accuracy of depth estimation through 12 image samples and analyzed the effectiveness of the generated spatial audio through 10 human participants. We achieved 94% average accuracy in depth estimation and observed that 10 participants correctly identified objects’ motion and location solely through auditory feedback with low latency.  These findings suggest that the approach provides a scalable and accessible model to restore independence for the visually impaired.

INTRODUCTION (needs conclusion!!!)

The estimated number of people affected by visual impairment around the world is 2.2 billion or more (1). Thus, the initiative of creating efficient assistive technologies is critical for public health (1). This immense global burden is not equally shared and disproportionately impacts rural areas, elderly people, and low- and middle-income countries (LMICs), thereby creating major barriers to human development, safe transport, and economic production at the macro-level (1, 2). About 2.5 billion people in the world currently require at least one type of assistive device, but 90% of the people with disabilities in resource-limited regions lack access to these devices (3). Social and financial exclusion, along with an increased reliance on family support, are the prevalent consequences of the dramatic discrepancy in accessibility (3, 4).  Key design factors to consider when designing assistive technologies for blind and visually impaired (BVI) people include characteristics of the target population and how these users interact with the environment (5).

Traditional white canes have served as the primary tools for navigation for the visually impaired (6). Soon enough, obstacle detection sensors were integrated into these white canes, making them smarter (7). Although they were very easy to use, they came with limitations such as failing to detect hazards within their immediate reach and the inability to provide enough information about the surroundings for the users, which are directly linked to independent mobility (6, 7). Independent mobility and security are vital components in an individual's life; lacking them limits access to social opportunities, work, and schooling in many cases (8). In modern assistive technology (AT) development, the focus has been on the design and development of Electronic Travel Aids (ETAs) to provide visually disabled persons with independent navigation and interaction with their environment (9). One of the fundamental principles in the engineering science of this kind is sensory substitution - where information collected using one modality (or sense) such as vision is reinterpreted into another (10). However, accurately translating rich and dynamic visual scenes into real-time sensory feedback is still a challenge today, which we are trying to overcome (10).  With the advent of deep learning, new methods based on computer vision can perceive interacting elements and obstacles in outdoor and indoor environments in real time for navigation (11).

We hypothesized that vision can be supplemented by transforming surrounding scene into depth map and representing them through time-synchronized spatial audio. Recent trends in developing navigation technology for the visually disabled no longer focus solely on auditory representation, but also incorporate stereoscopic vision, depth sensing, and three-dimensional spatial audio (12). This can be especially useful for approximately 43.3 million people who are completely blind and 295.1 million people living with moderate-to-severe visual impairment (MSVI) worldwide (13). To truly test the effectiveness of our hypothesized solution, we made a full working prototype of a system of a professionally made client-side device and a server. We named the project ‘Pixelsense’. The main feature used stereoscopic cameras to generate depth map of the surroundings continuously, then convert it into a 2 second frame of audio for auditory representation. To keep the device practical and comfortable to use while testing in real life, we offloaded the stereo image rectification, depth map estimation and harmonic audio sample generation tasks to a Python based server.

RESULTS

Accuracy of Depth Estimation

The accuracy of depth maps generated from raw image pairs is crucial because reliability of depth estimation directly affects the effectiveness of spatial sound generation. We took 12 image samples with our stereoscopic system and generated 12 depth maps corresponding to the images. For each image, we took 3 values from the depth matrix and compared the value with our measured distance from the camera lens to the physical point using a measuring tape.

Accuracy=Estimated depthActual depth×100%

The accuracy of each estimated distance was calculated through the Equation X. We showed the 3 estimated depth and their individual accuracy from one sample image (Figure 1).

IMPORTANT: ABOVE IMAGE NEEDS TO BE REPLACED WITH RECENT, PROFESSIONAL ONE AND DESCRIPTION NEEDS TO BE ELABORATED. Figure 1. Depth estimation from image. Left image shows the actual image with the actual depts of 3 points. Right image shows the color-coded depth map with estimated distance, error, and accuracy of three target points corresponding to the Left image’s 3 points.

Based on the data we collected, it was evident that the accuracy of depth estimation was very high for short ranges (typically 94% accuracy for 5cm to 150cm) and slowly dropped as distance increased (Figure 2).

Any accuracy over 80% was perfectly usable and reliable for spatial audio generation, suggesting that our system was reliable for 5cm to 300cm range (Figure 2).

Figure 2. Estimation accuracy vs distance graph. The graph is based on the 36 depth samples from a wide range of distance (25cm to 600cm). Each point on the line represents the expected accuracy (y) for measuring a distance (x). The very small negative slope of the line indicates negligible accuracy drop with increased distance.

Effectiveness Spatial Audio Generation

After each depth map is generated, they are converted into stereo audio for the user.

(Equation 5)

To transform the depth map into audible sound, X, Y, and Z axes of the visual field is correlated to time/panning, pitch and volume respectively (14). To determine the effectiveness of the audio representation of the depth map for navigation and environmental understanding, we let our device be used by 10 visually impaired participants. Each of them, wearing our device was assessed through three tests: determining an object’s position, determining an object’s motion and safe navigation. Except for the safe navigation test, the other two tests were taken with a red ball as an object. In safe navigation test, the participants were taken to an unknown room, and were told to get out of the room solely depending on the audio feedback. We showed the test results of all participants in (Figure 3).

| Participant ID | Age | Sex | Test Results |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Trial No. | OPD (second) | OMD (second) | SN (second) |
| P1 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.6 |  |
| P2 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P3 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P4 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P5 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P6 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P7 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P8 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P9 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| P10 | 22 | M | 1st | 1.5 | 1.8 | 12 |
|  |  |  | 2nd | 2.0 | 1.5 |  |
|  |  |  | 3rd | 1.7 | 1.7 |  |
|  |  |  | Mean | 1.73 | 1.67 |  |
| Mean |  |  |  | 1.84 | 2.1 | 14 |

OPD – Object Position Determination, OMD – Object Motion Determination, SN – Safe Navigation

Figure 3. Test result of participants. Participants successfully pointed towards the position of a red ball held by another person’s hand in the air in all three trials with a mean response time of 1.84s. Participants also uttered the motion of a red ball in the air (moved by another person) verbally (‘left’, ‘right’, ‘up’ etc.) within a mean time of 2.1s after movement. The participants also successfully got out of a room within 14s. All were done solely through stereo spatial audio cues.

DISCUSSION

Our results supported that continuous depth map estimation of surroundings with high accuracy (> 90%) using cheap camera modules and embedded systems is indeed possible and reliable by offloading computationally expensive tasks to a server (Figure 2). The survey and tests on 10 visually impaired participants exhibited positive results as the participants could determine position and motion of objects solely through spatial audio cues generated by our device. In our sensory substitution pipeline, the spatial audio generated for each depth map or image comes out in a 2 second time window of audio samples. The resultant mean response latency of only 2.1s for Object Motion Determination (OMD) and 1.84s for Object Position Determination (OPD) are highly significant as these align almost perfectly with the 2 second sweep duration (Figure 3). This shows that participants were able to determine object’s motion and position within the first audio scan cycle. This suggests that mapping the vertical coordinate to major pentatonic scales, using S-curve function for horizontal mapping and mapping distance to exponential amplitude (sound volume) provides an intuitive cognitive model. Moreover, the safe navigation (SN) test showed that participants were able to navigate with minimal effort indoors.

The decrease in depth estimation accuracy with distance (94% at 5cm to 300cm range and 70% at 500 to 600cm range) is not a flaw of our Semi-Global Block Matching (SGBM) or Weighted Least Squares (WLS) post filtering algorithms, but a direct outcome of mathematical limits of stereoscopic triangulation. As baseline distance between the camera lenses are fixed at 59mm for keeping the wearable device compact, pixel disparity changes become exponentially small and eventually flatten out as distance increase.

Apart from that, stereoscopic depth estimation works best for parts of image with non-repetitive textured surface, and the accuracy can drop significantly for parts of images that have repetitive texture or no texture at all, which is a limitation of our system (15). Furthermore, as we are using cheap, passive cameras with plastic lenses, they lack active illumination (like LiDAR or infrared cameras), which degrades depth estimation accuracy in low-light conditions. Our system focuses exclusively on visual-auditory sensory substitution, which can limit user’s spatial awareness by bottlenecking user’s audio channel. Future improvements on our research could potentially bypass this limitation by fusing audio cues with haptic  feedback to achieve improved navigation (16).

The results gathered from our cohort of ten visually impaired participants provided excellent initial proof-of-concept validation. But the small sample size, despite of multiple trials per individual limited our ability to showcase extended statistical significance on a broader demographic. Furthermore, our testing setup was carried out in controlled and indoor environment, which masks the unexpected problems and results that could be found in more dynamic and real-world setups such as bustling streets or densely crowded public spaces. As our users had experienced the technology for not more than two hours including tests, our current results did not account for long term physiological and neurological factors that manifest over time. To extend this research, several units of our device will be required to manufacture for testing a demographically diverse population in uncontrolled, dynamic environment over extended period and determine the learning curve of understanding spatial audio cues.

Our investigation into the efficiency of Pixelsense puts up several scientific questions, one of which is to explore the limits of edge computing on resource-constrained hardware for better accessibility of wearable assistive technology offline. In this regard, deep neural network (DNN) based models with quantization may be used to run depth estimation process locally with minimal power consumption. Future works should focus on incorporating multisensory feedback by incorporating haptic actuators. Finally, we plan to address instability of head-mounted cameras by adding Inertial Measurement Units (IMU). This will allow us to tackle problems that happen in dynamic environments, such as motion blur which can produce incorrect depth estimations.

Our development and evaluations demonstrated that integrating affordable, low-cost embedded systems architecture with server-side computer vision provides a highly viable paradigm for wearable assistive technologies. Ultimately, by prioritizing open computational architecture and compactness, this system provides the foundational and scalable framework for democratizing advanced navigation aids.

MATERIALS AND METHODS

Hardware Configuration and Integration

The physical device, which is a piece of technology attached to a spectacle frame, is one of the core parts of Pixelsense. This device contains a 3D-printed (printed using PLA filament) frame which houses the main circuit board and electronic components. The two OV3660 3-megapixel cameras are placed on top of the frame housing, and the base to which they are attached is kept as flat as possible to maintain identical vertical and forward alignment for both sensors. The two cameras are placed on the same horizontal axis with their optical centers 59-mm apart.

[

2 images of the device:

1. The full, intact device with frame

2. Only the front and back side of the circuit board

]

The device contains two independent but connected systems, which are centered around the Seeed Studio XIAO ESP32 S3 Sense (17). Each of these boards contains the necessary ESP32-S3 microcontroller unit (MCU) and a Flexible Printed Circuit (FPC) connector (to which we connect the cameras via an FPC cable). We soldered the two ESP32 boards onto a custom-designed Printed Circuit Board (PCB).

A MAX98357A (18) chip-based board, which integrates a digital to analog converter (DAC) and a mono audio amplifier, is connected to each of the ESP32-S3 board via Inter-Integrated Sound (I2S) protocol. These take in digital audio samples, modulate the samples into analog signals, amplify them, and play them via the earphones.

A single VL53L0X chip-based time of flight (TOF) infrared-laser distance sensor (19), which communicates with the ESP32-S3 via Inter-Integrated Circuit (I2C) protocol, is soldered onto the PCB. This sensor is used to quickly detect any obstacles within 1-meter range.

We have also added two TTP223 chip-based capacitive touch sensors for the user to activate different features and modes of the device (20). A 22pF capacitor is soldered to each of the touch sensors for better touch sensitivity.

The two ESP32-S3 MCUs communicate with each other via Universal Asynchronous Receiver-Transmitter (UART) protocol, which allows these two independent systems to send synchronous image streams (captured by the two cameras) to the server and receive audio samples from the server to play them through the stereo speakers synchronously.

Server Configuration and Integration

The server, for our testing is a consumer laptop. The server handles all the complex and computationally expensive tasks like rectifying stereoscopic images, generating pixel disparity and depth map and generating stereo audio samples. The program within the server that handles everything is written in Python 3.10.

It utilizes Transmission Control Protocol (TCP) to establish a connection with the ESP32-S3. Whenever a user touches a touch sensor, the activation command associated with the gesture is sent to the server via TCP. Whenever the device turns on, the two ESP32-S3 individually establish the connection with the server. This way, the server knows which feature to activate. Moreover, User Datagram Protocol (UDP) is utilized to send and receive image and audio sample streams with minimal network overhead. Here, websockets 15.0.1 (21) library was used for handling TCP connection and socket library (22) was used to handle UDP connection. OpenCV 4.10 was used for camera calibration, stereo rectification and depth estimation.

Depth Map Estimation and Spatial Audio Generation

(Equation 1)

This feature allows a visually impaired individual to perceive the objects and their motion through melodic frequencies of sound. To activate this feature, the user presses a touch sensor. The ESP32 detects it via its digital pin and sends a message to the server for the initialization of the feature. The two ESP32 boards coordinate with each other via UART and stream images of the user’s surroundings to the server synchronously, making sure every pair of image frame captured by the cameras are taken at the same time. The server, receiving the image streams first rectifies the pair of images with the OpenCV’s stereoRectify method (23). To rectify the image pairs, first we obtained the intrinsic and extrinsic parameters of the camera by the chessboard calibration method with 40 image samples taken at diverse distance and angles for better accuracy and to minimize stereo reprojection error (24).  According to the pinhole camera model, a point in 3D space, P=X,Y,ZT is projected onto a 2D plane at pixel coordinates u,vT. We use the intrinsic camera matrix (K) to represent this relation:

suv1=KXYZ

(Equation 2)

In Equation 1, s is a scaling factor (representing depth). The intrinsic camera matrix K for each of the cameras has the following form, which contains the focal lengths in pixels (fx.fy) and principal points (cx,cy):

K = fx0cx0fycy001

Cameras, especially the OV3660 camera which contains a plastic lens is prone to radial and tangential distortion. To solve this, we obtained the distortion coefficient vector D=k1,k2,p1,p2,k3 during camera calibration, where k1,k2 and k3 are radial distortion coefficients and p1 and p2 are tangential distortion coefficients. These intrinsic parameters (camera matrix and distortion coefficient vector) are obtained through OpenCV’s calibrateCamera method for each of the two cameras.

(Equation 3)

Let Pl and Pr are two points viewed by the left and right cameras respectively, which corresponds to the same 3D point in the scene. The geometric transformation between the optical centers of two cameras is represented by a 3x3 rotation matrix (R) and a 3x1 translational vector (T), representing the physical distance between the two cameras’ optical centers:

Pr=R⋅Pl+T

By utilizing OpenCV’s stereoCalibrate method, we solved for the rotation matrix and translational vector, which are the extrinsic parameters. We stored the extrinsic and intrinsic matrices as a JavaScript Object Notation (JSON) file in our server after calibration.

By feeding these extrinsic and intrinsic matrices to initUndistortRectifyMap method of OpenCV, and remapping the images, a 3D point P was projected onto the left and right rectified images such that the vertical coordinate is identical.

(Equation 4)

During image stream, every single image is rectified and then goes through multiple algorithms and pipelines of disparity estimation, post-filtering optimization, and geometric transformation. Every pair of images are converted into grayscale and Semi-Global Block Matching (SGBM) algorithm is applied (15). Through this algorithm, two raw pixel disparity maps are generated for the left and right cameras. To enhance smoothness of the output, a Weighted Least Squares (WLS) fast-global filter is then applied to the raw left disparity map, using the left grayscale image as a structural guide to refine boundaries (25). According to our tests, a filtering parameter λ= 8000 and a color sensitivity parameter σcolor=1.5 worked the best. The resulting filtered sub-pixel disparity map is downscaled by a factor of 16.0 to reconstruct the true pixel disparity (d). The absolute pixel distance Z (in millimeters) of each pixel coordinate is calculated using triangulation across similar triangles. The absolute depth map is generated by applying a simple formula for each

(Equation 5)

value from the pixel disparity map:

Z=f⋅Bd

Here, f is the focal length of the camera sensor in pixels, extracted from the camera matrix element K0,0. B is the baseline distance between the optical centers of the two cameras derived from the absolute horizontal translation vector component Tx. d represents the true pixel disparity computed via the WLS filter.

To transform the depth map into audible sound, X, Y, and Z axes of the visual field is correlated to time/panning, pitch and volume respectively (14).

[Show a depth map with x, y, z labeled with pitch, time and volume]

To represent vertical position of an object, the vertical axis is divided into 20 horizontal bands and each band is mapped to a unique audio frequency. To provide pleasant listening experience, a Major Pentatonic Scale is used which begins at a base frequency of 130Hz and is capped at 3kHz to prevent audio distortion. Higher frequency corresponds upper parts of the depth map, while lower frequencies indicate lower objects. The horizontal field of view is represented by “scanning” the image (depth map) from left to right over a definite period. The time of one frame scanning is flexible and can be set by the user. We used a 2 second time frame for testing. As the scan progresses from left to right, the audio signal is distributed between the left and right stereo speakers using an Equal Power Panning technique. To keep the panning natural, it follows an S-curve function:

Sp=1-cosπ⋅p2

(Equation 7)

(Equation 6)

Here, p represents the progress of panning in 0 to 1 range.

ALp=1-Sp

ARp=Sp

The amplitude for the left and right channel is given by the functions ALp and ARp respectively.

The proximity of the objects (Z) is directly represented by the amplitude of the audio signal. At first the values of depth map are normalized against a predefined maximum distance (Zmax) to get a range of 0 to 1. The normalized depth values are raised to the power of two. This ensures objects close to the user sound louder compared to insignificant background noise.

Determining Accuracy of Depth Estimation

(Equation 8)

The accuracy of depth map was calculated by comparing the actual depths (distance from the camera lenses) and the measured depth at different points of sample scenes. We kept the device stable on a stand. Before capturing an image, we determined the distance from the camera lens to some objects, corners, and points of the scene by using a measuring tape. This way, we got the actual depths of some points of a scene. After that, we captured an image of that scene with the device and obtained the depth map matrix. By utilizing a python script, we created a graphical user interface which allowed us to easily fetch the estimated depth of every point of the image from the depth map.

Accuracy=Estimated depth × 100%Actual depth

The accuracy of measurement was determined through the (Equation 8). In this way, we determined the accuracy of depth estimation for a total of 36 points from 12 image samples. The average accuracy turned out to be 94%. The range of distance measured and estimated during accuracy determination was 10 cm to 300 cm.

Determining Effectiveness of Spatial Audio

The effectiveness of spatial audio was determined by observing how users avoided hazards and perceived the motion and position of objects. We let our device be used by 10 visually impaired individuals. The people were given one hour to get familiar with the spatial audio cues. After that, 3 tests were given to the participants. The first one was to determine the position of a red ball by pointing towards it. The ball was in midair held by a person’s hand. In the second test, a person moved a red ball in the air in specific directions. The participant had to tell the direction of movement by describing it vocally. In these two tests, we noted down how fast the participant responded in a notepad. In the final test, the participant was taken to a room, from which he had to escape solely depending on audio cues generated by our device. We noted down the time taken for the participant to escape.

REFERENCES

1. ‘World report on vision’ World Health Organization, https://www.who.int/publications/i/item/9789241516570, Accessed 6 July 2026.

2. ‘Vision Impairment is a $1 Trillion Productivity Problem: What Can Governments Do About It?’ Center For Global Development, https://www.cgdev.org/blog/vision-impairment-1-trillion-productivity-problem-what-can-governments-do-about-it, Accessed 6 July 2026.

3. ‘Global Report on Assistive Technology’ 1st ed. World Health Organization (WHO) and United Nations Children’s Fund (UNICEF), Geneva, 2022.

4. United Nations Department of Economic and Social Affairs ‘‘Disability and Development Report 2018: Realizing the Sustainable Development Goals by, for and with Persons with Disabilities’’ United Nations, Erscheinungsort nicht ermittelbar, 2019.

5. Pawluk, Dianne T.V., Adams, Richard J., et al. ‘Designing Haptic Assistive Technology for Individuals Who Are Blind or Visually Impaired’. IEEE Transactions on Haptics, vol. 8, no. 3, 1st July 2015, pp. 258–278, doi:10.1109/TOH.2015.2471300.

6. De Paz, Carlos, Huertas, Juan Antonio, et al. ‘Enhancing navigation and obstacle avoidance with a vibrotactile device as secondary electronic travel aid’. Disability and Rehabilitation: Assistive Technology, vol. 20, no. 4, 19th May 2025, pp. 1140–1150, doi:10.1080/17483107.2024.2417264.

7. Mahdy, Shimaa, Yousri, Ibrahim, et al. ‘Smart Blind Stick using Mobile Application and Bluetooth Module’. International Journal of Industry and Sustainable Development, vol. 6, no. 1, 1st Jan. 2025, pp. 43–53, doi:10.21608/ijisd.2024.307950.1079.

8. Gupta, Yukti ‘Assistive Technology for Visual Impairment: Innovations, Impact, and Inclusion’. International Journal For Multidisciplinary Research, vol. 7, no. 6, 27th Nov. 2025, pp. 61850, doi:10.36948/ijfmr.2025.v07i06.61850.

9. Zhang, Xiaochen, Huang, Xiaoyu, et al. ‘Advancements in Smart Wearable Mobility Aids for Visual Impairments: A Bibliometric Narrative Review’. Sensors, vol. 24, no. 24, 14th Dec. 2024, pp. 7986, doi:10.3390/s24247986.

10. Barontini, Federica, Bettelani, Gemma Carolina, et al. ‘A User-Centered Approach to Artificial Sensory Substitution for Blind People Assistance’. In Converging Clinical and Engineering Research on Neurorehabilitation IV, edited by Torricelli, Diego, Akay, Metin, et al., Biosystems & Biorobotics. Springer International Publishing, Cham, 2022, Vol. 28, pp. 599–603.

11. Salman Shah, Syed, Imran, Abid, et al. ‘Vision-Based Smart Wearable Assistive Navigation System Using Deep Learning for Visually Impaired People’. Automation, vol. 7, no. 2, 1st Mar. 2026, pp. 41, doi:10.3390/automation7020041.

12. Gupta, Yukti ‘Assistive Technology for Visual Impairment: Innovations, Impact, and Inclusion’. International Journal For Multidisciplinary Research, vol. 7, no. 6, 27th Nov. 2025, pp. 61850, doi:10.36948/ijfmr.2025.v07i06.61850.

13. Ying, Gui-Shuang ‘The Global Burden of Blindness and Visual Impairment: Progress and Challenges’. Ophthalmic Epidemiology, vol. 33, no. 1, 2nd Jan. 2026, pp. 1, doi:10.1080/09286586.2025.2580766.

14. Meijer, P.B.L. ‘An experimental system for auditory image representations’. IEEE Transactions on Biomedical Engineering, vol. 39, no. 2, Feb. 1992, pp. 112–121, doi:10.1109/10.121642.

15. Hirschmuller, H. ‘Stereo Processing by Semiglobal Matching and Mutual Information’. IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 30, no. 2, Feb. 2008, pp. 328–341, doi:10.1109/TPAMI.2007.1166.

16. Lloyd-Esenkaya, Tayfun, Lloyd-Esenkaya, Vanessa, et al. ‘Multisensory inclusive design with sensory substitution’. Cognitive Research: Principles and Implications, vol. 5, no. 1, Dec. 2020, pp. 37, doi:10.1186/s41235-020-00240-7.

17. ‘Getting Started with Seeed Studio XIAO ESP32-S3 Series’ Seeed Studio Wiki, https://wiki.seeedstudio.com/xiao_esp32s3_getting_started/, Accessed 5 July 2026.

18. ‘MAX98357A/MAX98357B - Tiny, Low-Cost, PCM Class D Amplifier with Class AB Performance’ Analog Devices, https://www.analog.com/media/en/technical-documentation/data-sheets/max98357a-max98357b.pdf.

19. ‘World’s smallest Time-of-Flight ranging and gesture detection sensor’ STMicroelectronics, https://www.pololu.com/file/0J1187/vl53l0x.pdf.

20. ‘1 KEY TOUCH PAD DETECTOR IC’ Tontek Design Technology, https://files.seeedstudio.com/wiki/Grove-Touch_Sensor/res/TTP223.pdf.

21. ‘websockets’ websockets, https://websockets.readthedocs.io/en/15.0.1/index.html, Accessed 5 July 2026.

22. ‘socket — Low-level networking interface’ Python documentation, https://docs.python.org/3/library/socket.html, Accessed 5 July 2026.

23. Bradski, Gary and Kaehler, Adrian ‘‘Learning OpenCV: Computer Vision with the OpenCV Library’’ O’Reilly Media, Inc., 24th September 2008.

24. De la Escalera, Arturo and Armingol, Jose María ‘Automatic Chessboard Detection for Intrinsic and Extrinsic Camera Parameter Calibration’. Sensors, vol. 10, no. 3, Mar. 2010, pp. 2027–2044, doi:10.3390/s100302027.

25. Min, Dongbo, Choi, Sunghwan, et al. ‘Fast Global Image Smoothing Based on Weighted Least Squares’. IEEE Transactions on Image Processing, vol. 23, no. 12, Dec. 2014, pp. 5638–5653, doi:10.1109/TIP.2014.2366600.

Appendix

GitHub link to the server and microcontroller firmware: https://github.com/ExaSukiAS/pixelSense